140 points (70 short questions (14 questions)/ 70 arithmetic tasks (3 tasks))

Fragenteil:

- 4 Specify requirements (top or further req.)

- Explain "Stability Measure" and name a point that is from "immutable" (i.e. aircraft config) and one that can differ from mission to mission

- 3 types of flow separation and which is the most critical regarding stable

- Differences Engine over / under Wing with regard to different aspects (tick which one applies to which config):

  - Cabin Noise

  - Maintainability

  - Risk of damage because things are sucked in

  - Noise on Ground

  - Interference during flight

- 2 Identify the difficulties encountering or hampering the introduction of full electric airplanes in the long-range sector

- L/D or D/veocitiy(?) Diagram:

  - Draw where induced drag, total drag and zero lift drag

  - I.what to draw with SEP

  - Draw optimal L/D ratio (but was believed drag regarding speed)

- Assign terms & explanation: what is net thrust, static thrust, gross thrust, insalled thrust, specific impulse, etc.

- What influences MTOW, one point each regarding Atmospheric, Infrastructure Airfield, Plane Configuration before Start

- Design chart given and record how 2 curves behave when a profile is used for higher Mach numbers (one vert. Curve was i.was regarding landing and the other ran from the top left towards the landing curve, as a curved course)

- Difference between profile or wings for low/high Mach numbers

- Given was a 3-page view of a transport aircraft for transonic areas. 4 aspects that suggest that it is a transport aircraft for transonic areas.

- Draw typical flight envelope and enter 2 loads (nz vs. speed). Point at high speeds enter which with a small angle of attack change the structural (?) Loads. Assume a short, static maneuver

- Indicate when Polhamus, Lifting-Line and Slenderbody are used (on the left the terms, on the right the "explanation" with Ma number and i.what else. Then assign them correctly

- Explain winglet function by means of 2 sketches (once top view on aircraft) and once the winglet (profile)

Calculation tasks:

- Aircraft config given, as in exercise (1)

- Task

- Beziehung zw. Maximum Take-Off Weight, Fuel Weight, Payload-Weight und operational empty weight angeben

- Diagram given and thus iteratively estimate an MTOW

- Task

- Wings given with single-slot flap and various diagrams

- Calculate deltaCl at an angle of attack of x°

- Calculate Cl at an angle of attack of x°

- Calculate air density at h=x meters

- Calculate maximum MTOW

- Task:

- On and front view (curve flight i.e. with roll angle) given by aircraft

- Draw trust, weight, centrifugal, and lift vectors in both views

- Calculate centrifugal force

- Calculate Equilibrium point

- Wings with constant buoyancy distribution given, calculate bending moment